(All rights reserved)

SCHOOL OF ENGINEERING SCIENCES

DEPARTMENT OF BIOMEDICAL ENGINEERING

COURSE CODE AND TITLE: BMEN416 – TELEMETRY AND TELEMEDICINE  LAB REPORT ONE (1)

GROUP  MEMBERS

| NAME | ID |
| --- | --- |
| DELORES NKRUMAH | 10956674 |

DATE: FRIDAY, July 9, 2025.

INTRODUCTION

Telemetry is the technology of remotely measuring and transmitting data from one location to another. In healthcare, telemetry plays a crucial role in telemedicine, which encompasses the use of information and telecommunication technology to render medical services irrespective of the geographical distance between physicians and patients. Telemedicine has been in practice as far back as the 1900s. It covers any form of electronic communication (both wired and wireless) between health workers and patients from a remote location. Telemedicine enhances access to healthcare, especially in rural or underserved areas, and improves the efficiency of diagnosis and treatment. This also greatly reduces the cost of medical diagnosis and the need to travel long distances for both the patient in search of professional consultations and healthcare institutions in the form of ambulances.

In recent years, more focus has been placed on the deployment of wireless communication technologies for various telemedicine systems and designs. These include ZigBee, Bluetooth, VSAT, WLAN, WiMAX, GPRS, and UMTS (3G and 4G)

Sensors are fundamental to both telemetry and telemedicine. They are used to capture real-time physiological data such as heart rate, body temperature, blood pressure, and blood oxygen saturation (SpO₂). The integration of sensors into telemedicine systems is essential for accurate, real-time monitoring of vital signs such as heart rate and SpO₂. Wireless sensors, in particular, enhance patient mobility, reduce hospital admissions, and enable continuous remote monitoring, especially in home-based care.

The importance of wireless monitoring and the role of wireless sensors become apparent in the wake the COVID-19 pandemic, as it ensured continued patient care, reduced physical contact and limited the spread of infection.

This lab aims to demonstrates how telemetry principles can be implemented in a small-scale system by simulating an SpO₂ sensor with the ESP32 microcontroller. The system uses Bluetooth communication to wirelessly transmit simulated SpO₂ readings to a connected device, mimicking a real-world telemedicine scenario.

METHODOLOGY

- An ESP32 board was programmed using the Arduino IDE to simulate a wireless SpO₂ monitoring system.

- The BluetoothSerial.h library was used to enable Bluetooth communication.

- The ESP32 was initialized with the name "ESP32_O2Sensor" using SerialBT.begin().

- In the loop, the code checked for a Bluetooth connection using SerialBT.hasClient().

- If connected, a random SpO₂ value between 92% and 100% was generated using random(minSpO2, maxSpO2 + 1).

- The value was formatted as a string (e.g., "SpO2: 96 %") and sent via Bluetooth using SerialBT.println().

- The same value was printed to the Serial Monitor for verification.

- A 2000 ms delay controlled the data transmission interval, and the process repeated continuously.

RESULTS

DISCUSSION

The biological parameter selected for this lab was SpO₂ (peripheral capillary oxygen saturation),  the percentage of hemoglobin binding sites in the blood that are occupied by oxygen. In simple terms, SpO₂ indicates how well oxygen is being carried from the lungs to the body’s tissues. This parameter is one of the most critical indicators of respiratory and cardiovascular health. SpO₂ is commonly measured using a non-invasive optical sensor, known as a pulse oximeter, which analyzes light absorption in blood vessels to estimate oxygen saturation.  Under normal physiological conditions, SpO₂ levels range from 95% to 100%, indicating healthy oxygen transport from the lungs to the body's tissues. Readings below 92% are typically considered abnormal and may signal hypoxemia, a condition where insufficient oxygen is reaching vital organs, and levels falling into the mid-80s or lower may lead to tissue hypoxia and serious symptoms such as organ failure, cyanosis or death.

The decision to focus on SpO₂ in this lab was intentional due to its clinical importance and widespread use in both hospital and telemedicine settings. Monitoring SpO₂ is especially critical for patients with chronic obstructive pulmonary disease (COPD), asthma, heart failure, pneumonia, and those recovering from surgeries or infections such as COVID-19. In such cases, early detection of a decline in SpO₂ can lead to prompt intervention, improving patient outcomes. Moreover, SpO₂ is often included in remote patient monitoring (RPM) kits for home use, making it a highly relevant parameter for telemetry and telemedicine applications.

Several design decisions in the code were made to ensure realism and clinical accuracyy. The SpO₂ value range was set between 92% and 100%, aligning with established physiological thresholds. This range captures the spectrum from mild desaturation to optimal oxygenation without including dangerously low values that would require immediate intervention. The transmission interval was set to 2000 milliseconds (2 seconds), which balances timely updates with efficient use of bandwidth and system resource. The code used random(minSpO2, maxSpO2 + 1) to generate realistic, evenly distributed SpO₂ values that mimic natural fluctuations seen in real sensors, making the simulation suitable for developing remote monitoring systems.

CONCLUSION

This lab successfully demonstrated the simulation of a wireless SpO₂ monitoring system using the ESP32 microcontroller and Bluetooth communication. By generating and transmitting biologically realistic oxygen saturation values, the system modeled how modern telemetry and telemedicine platforms collect and send vital health data remotely. While telemedicine is not a complete substitute for physical diagnostics and hands-on medical care, it plays a vital complementary role by reducing hospital crowding and helping to control the spread of infectious diseases.

REFERENCES